USB TO RS232 INSTALLATION GUIDE

Set up 1

Set up from software

- Through  CD-ROM find adaptable OS .

- Install  CP210x_VCP_Win2K_XP_S2K3 software

- Choose NEXT”

4.Choose ”ACCEPT”

5.Cjppse destination location : ( Initial value ) then choose next step.

6.Choose ”Install”

7.Finish

8.Will be produced contents and files in C:\SiLabs.

9.Get in to C:\SiLabs\MCU\CP210x\Windows

10. administer CP210xVCPInstaller

11.Choose ”route (Initial value ) and Install”

12.Start Scanned hardware ( Don’t set up hardware now)

13.Will be turn off after finish set up

14.Will be find automatically when insert the hardware as below. ( Can check

device manager under control panel. )

15.Connect port have a new Com port , is meaning finish all the installations.

Setup step 1

Set up from hardware

Hardware set up step.

- Insert hardware

- Choose cancel

3. Through  CD-ROM find adaptable OS .

4. Install CP210x_VCP_Win2K_XP_S2K3 software

5.Choose ”NEXT”

6.Choose ”ACCEPT”

7. Choose installtion ”route (Initial value ) and Install”

8.Choose ”Install”

9.Finish

10.Start control panel

11.Get into system

12. .Start control panel

13.Will see CP2102 USB to UART( Hardware can’t use now )

14. Choose CP2102 USB to UART program.

15.Restart installation driver.

16..Choose ”Yes ” then press next step.

17.Choose list or location then press next step.

18.Point to “C:\Silabs\MCU\CP210x\Windows_2k_xp_s2k3 “ then press next step.

19.Start searching hardware.

20.Finish

21.Will be install again after finish set up.

22. Choose “ Yes : then press next step.

22. Choose list or location then press next step.

23.Point to “C:\Silabs\MCU\CP210x\Windows_2k_xp_s2k3 “ then press next step.

24.Start searching hardware.

25.Finish

26. .Start control panel

27. .Start control panel

28.Device manager will show CP210X USB .

29. Connect port have a new Com port , is meaning finish all the installations..